Amit Chowdhury

M.Tech’24 , IIT Jammu

LoRa :

Learned about what LoRa is and how to get location coordinates simply by coding using Arduino IDE with required libraries. Also implemented a project using Google Map API to visualize the current location on a Map. Code

LoRaMesher Library:

Learned what LoRa Mesher is !!

Through its github library implemented a project from the example section name: CounterAndDisplay using Visual Studio Code + Platformio. It will send a counter to anyone that can listen in one hop & the counter will be sent directly to one of the nodes inside the routing table. Each time it will change the destination.

To understand the source code gone through readme.md

Meshtastic Project:

Learned about Meshtastic Project.

Installed Meshtastic Firmware 2.1.23.04bbdc6 Alpha in the Lora module using cmd.

Useful References: 1 2 3 & YTvideos from Andreas Spiess and Ham Radio channel.

The commands used to install the firmware (Windows) are documented here.

Right after setting the region ‘IN’, connect the Meshtastic Application with LoRa using Bluetooth then set a communication channel.

Studied about Helium Network, TTN, NB-IoT.

Range testing with sf=12, the maximum reliable communication range achieved was 2.2km. Positioned one module at an elevation of 124 m above sea level on the IIT Jammu campus, while the other was placed on the car roof at an elevation of 80-110m above sea level. Send messages every 200-300m. The expected range should have been significantly greater, but it appears that the presence of small hills surrounding our campus, along with the second module being in a shadowed region, hindered my ability to establish communication.

Understanding & Modifying the source code of Meshtastic Project to connect a SuperNode with Wifi. Routing all the data packets from different nodes through that SuperNode.